◆ Causes for difficulty in engine starting and troubleshooting method

①Diesel engine has not run for a long time: fuel return pipe shall be under diesel level.

②Low pressure pipe has little air: expel air

③Crankshaft rotating speed signal, and camshaft signal are too weak, synchronization judgment time is too long: conduct adjustment again.

④Environment temperature is too low, pre-heating device is in failure: replace the pre-heating device

⑤The quality of diesel and oil is too bad to reach the standard: replace it with standard diesel or oil

⑥Gear breaks in starter or flywheel gear rings tooth interference: replace starter and flywheel gear ring.

⑦Piston rings and cylinder sleeves are worn out or valve sealing is not tight: Replace piston rings, cylinder sleeves or valve seat, valve

⑧Exhaust brake butterfly valve is seized to cause block of exhaust: replace the butterfly valve

Inner diameter of fuel pipe

Allowed fuel pipe length

Allowed pressure

Air inlet pipe of fuel tank

Air return pipe of fuel tank

⑨Fuel inlet/return pipes of the fuel tank cannot meet the design requirement for inner diameter and pressure