3.2 water temperature, oil temperature, air intake temperature are too high

When water temperature, oil temperature, air intake temperature are too high, ECU will enter into overheat protection function to limit engine power.

◆Causes of high water temperature and troubleshooting method

①Water level in water tank is too low: check that there’s water leakage, and add water when necessary.

②Water tank is blocked: check water tank, and clean or repair it

③Water pump belt is loose: adjust tensioning force as specified

④ Water pump shim is damaged, water pump impeller is worn out: check and repair or replace it

⑤Thermostat failure: replace it

⑥Water pipe sealing members are damaged, and air enters: check water pipe, joint, shim, etc, and replace the damaged members.

◆Causes of high oil temperature and troubleshooting method

①Oil level in the oil sump is low or the oil sump is in lack of oil; check the oil level and oil leakage places. Repair it and add oil

②Water temperature is too high: check the above causes for high water temperature and eliminate it

③ Coolant flows unsmoothly in oil cooler; check and clean it

◆Causes of high air intake temperature and troubleshooting method

Check heat dissipation ability of intercooler