1.1 Starter stops working

Generally, starter of Euro III engine is controlled by ECU. Mainly check conditions in the neutral gear. And ECU outputs a current to drive the starter rely; after the relay is connected, the battery will drive the starter.

So there are some factors during checking: neutral switch, start relay, battery, relevance of shutdown switch under vehicle.

Check if vehicle stays at neutral position

◆Check location of shutdown switch under vehicle (at OFF position)

Parking switch under vehicle

◆Check if neutral switch and wiring are in good condition, and try to use emergency start (press ignition switch continuously for 5 seconds)

Neutral position switch

Transmission

◆ Check if battery voltage is too low to start.

◆ Check if starter relay and wiring are in good condition.

◆ Check if starter is burned out

起动机

Starter

◆ Check if ignition switch and start switch are damaged

点火钥匙

开关

Ignition key switch