3.1 Injector failure

Injector failure, generally, can be classified with mechanical failure and wiring failure

Mechanical fault: needle valve is seized. Because diesel contains much dirt or water enters the valves to cause corrosion, the needle valve is seized in the fuel injector, leading to movement failure.  (Note: ECU might not show error!)

Wiring error: due to vibration, worn-out, etc, harness is disconnected or directly placed on on the cylinder cover to ground. ECU may show error

◆ Idle speed is unstable, and hear abnormal engine sound；

◆ Judge it by cylinder cut-off fuel or high pressure fuel pipe sensing (use diagnostic tools for accelerating test judgment)

◆ Generally, diagnostic instrument can activate test function on injector.