1.4 Crankshaft signal and camshaft signal losing

Two rotating speed sensors are installed on diesel engine; one is located on outer side of flywheel shell and another on high pressure fuel pump.

Functions: crankshaft location sensor and cylinder identification sensor. Electric control injection timing is decided by the two sensors. In case diesel engine cannot be started, two signals are lost.

Camshaft revolution sensor

Revolution sensor of crank shaft

◆ The possible cause of losing two signals

①Sensor is damaged. Harness is short-circuited or open-circuited

②Sensors are not fixed well so that big or small gap (generally 1±0.5mm) exists between sensor and sensitive tooth.

Troubleshooting method: check if sensor is damaged, harness is connected well, or sensor is loose, etc.

◆After dismantling high pressure fuel pump and flywheel, the installation shall be carried out in strict accordance with relevant processes to ensure the signal in synchronization.