Engine abnormal sound

1. Timing gear chamber makes abnormal sound

There’re many factors that may cause abnormal sound of engine. If there’s obvious mechanical noise on the front of engine during running and this sound becomes obviously loud especially when compressor is filled with air normally, but obviously low when the pressure regulating valve is releasing air(compressor running without load), usually the drive mechanism of timing gear chamber malfunctions.  At this moment, it is noted that vehicle cannot run by force, and should be driven to a repair trench and dismantle oil pan and check if there’s foreign mechanical matters in the engine oil of the oil pan. If broken gear is found, it means gear of timing gear system is broken. Aim at timing gear chamber by illumination (flash light), and rotate flywheel of engine and observe gear breaking position. If other mechanical stuff is found, it is required to analyze position of stuff. At this moment, check on timing intermediate gear bearing and compressor bearing and injector pump driving bearing is specially needed, and in fact, timing intermediate gear and injector pump driving shaft support bearing easily encounters a fault “falling apart”. When finding failure position, dismantle and repair on failure position (see section “Engine Installation” for details) For old structure, use two 3 high speed bearings on timing intermediate gear, and the bearing cannot be replaced by common Chinese-made 206 bearing, otherwise, it will make frequently falling apart on bearing.

If continue driving the vehicle, damage will be worsened; and once crankshaft is damaged, replacement of crankshaft gear will become very difficult.

2. Air valve is making noise

It is easy to judge air valve noise because noise position is relatively obvious; especially when engine is running in low speed, noise is more obvious. The main reason of valve noise is valve gap. If a certain valve gap in cylinder has much difference from other valves obviously, it is required to check carefully if adjusting bolt is loose, valve spring is broken, or the valve lever is bent or broken.

3. Cylinder noise

There are many causes on abnormal noise of engine cylinder. First, injector spray is not good and it will make noise on cylinder, followed with smoke emission. Check the noise by fuel cut-off. Injector poor atomization will cause unstable engine running; long term poor atomization will also result in cylinder knocking incident because unburned diesel has flush function on lubrication of cylinder. In more serious case, oil amount in oil pan will increase gradually.

Inconsistent injection amount in injector pump will also cause cylinder knocking. Cylinder with obvious more injection amount will have cylinder knocking phenomenon. Fuel cut-off method can be used to judge which cylinder has too much fuel injection obviously. After reducing the cylinder fuel injection amount, (loosen cylinder injector joint by a spanner), cylinder knocking can disappear obviously.

Serious cylinder scuffing, cylinder sticking and piston ring seizure, etc can make cylinder knocking noise. Meanwhile, the fault is accompanied with crankshaft case gas carry-over; if single cylinder sound is obvious and gas carry-over occurred one by one string, which indicates that a certain cylinder is scuffing seriously. Sound noise caused by cylinder scuffing cannot be eliminated by fuel cut-off, but can be weakened.  In case of slight cylinder scuffing (cylinder piston is not damaged seriously, but just has a slight grinding and matching is not over usage limit), there is no need to change cylinder sleeve piston, and it is ok to repair it through replacement of piston ring. The engine repaired in this manner shall run under proper temperature and loading during run-in period, and then bear safe load after run-in period. In case of serious cylinder scuffing, cylinder sleeve, piston and piston ring subassembly must be changed.

Fig.1-2 Method of pulling cylinder sleeve

Replacement method is to dismantle cylinder cover with scuffing fault, remove the engine oil pan, dismantle linkage lever and pull out piston linkage subassembly. Pull out cylinder sleeve with special tools as shown in fig. 1-2, and dismantle piston subassembly from linkage. Choose piston and cylinder according to requirement of matching gap, clean cylinder hole and hole concave, then wipe hole and outer diameter grease by trichloroethylene, and smear Mos2 powder on the surface of cylinder, and try to choose the cylinder sleeve suitable for the gap of cylinder hole. In this way, it will be easy to press bush into hole by hand or simple tools(see section “Engine Installation for details). Check if the cylinder sleeve is higher than top surface of the cylinder block within range of 0.05～0.10. Heat the new piston subassembly (note that piston is matched with the engine model) to 80℃ for 10 minutes and then fit it with the linkage. Please note the installation location of the first and second air ring; surface of the ring marked with “TOP” or other letter mark must be upward. The 3rd ring’s opening shall be staggered with 120° before installation into cylinder sleeve and the ring opening shall be 30° out of piston pin axis. Remove the piston and the linkage subassembly from bush by special tools. Note the installation location of piston and linkage, location of linkage subassembly and cylinder sleeve. Tighten linkage bolts with specific torque Note that linkage bolt can be only used for one time, which means that linkage bolt must be changed each time when dismantling linkage. When cylinder cover, valve tappet and valve chamber cover are mounted, repair is finished. Specific consequence (see section “Engine Installation for details) and torque should be followed when mounting cylinder cover, and main and vise bolt of cylinder cover should be tightened.

When cylinder sleeve or piston is seriously worn out, and bush piston gap have exceeded limit will also make noise, and the sound cannot be eliminated by fuel cut-off. After decomposition, it is found that steps appear on cylinder mouth. At this moment, sleeve and piston subassembly should also be replaced.

If cylinder knocking happens after replacement of timing belt, the sound position is obvious and becomes loud with acceleration of engine, and note not to drive it any more. First, it is required to check mark of timing gear and timing mark, that is 1st cylinder TDC(1st cylinder TDC flywheel “TO” scale line should be aligned with the flywheel shell scale line, the 1st cylinder intake and exhaust valves should be closed). Check if the timing mark on camshaft gear is aligned with the scale line in the timing shell.  Even if the scale line misplaced for one gear, it will cause the sound of cylinder knocking, and will damage piston and valve in more serious case, and even serious incident of damage cylinder, etc.

If loud abnormal sound is made suddenly during running, it is required to stop running immediately, and conduct inspection carefully. Because obvious abnormal sound suddenly always results from damage of mechanical parts such as valve spring is bent, and valve drop into cylinder, valve bent or valve seat drop into cylinder, sleeve broken and even damage, linkage bolt broken causing drop of linkage cover, serious damage due to timing intermediate gear support fall apart. This kind of fault will lead to serious consequence, and should pay extra attention to it.

4. Bearing bushing sound

If main bearing bushing or linkage bearing bushing is worn out seriously or burnt out, it will make bearing bushing noise. Cause to common noise of bearing bushing is difficult to judge. The characteristic of linkage bearing noise is clear, and when engine is accelerating suddenly, the sound is obvious, and the sound is increased as rising of rotation speed and load. Fuel adding can be continued when checking, and there will be “dada” sound obviously, and it will be weakened with fuel cut-off method. The characteristic of main bearing bushing noise is that the noise is tediously, and becoming loud with acceleration of engine, and it is obvious when accelerate suddenly. Check it with fuel cut-off test, single cylinder fuel cut-off without obvious change, and two neighboring cylinder beside failure cylinder fuel cut-off sound is decreasing obviously. The main reason of bearing bushing noise (or worn out or burning) is that oil is too dirty, and impurities enter into shaft journal and bearing oil slick, thus bearing is worn out, and lack of oil, and fuel feed is not enough, and it is difficult to form enough oil membrane between axle hole and journal. In lubrication system, the gap of certain part is too much and lead to leakage of oil, lubrication is not good, the gap between the journal and the bearing bushing is too much, and engine overheat, etc. It is noted that WD615 series diesel engine is high intensified diesel engine, and it is required to use AP1CD grade and AP1CF grade(Euro II engine with 300 hp and Euro III engine) supercharge diesel engine (see section “Oil, Water and Power” ).This requirement should be followed without any compromise.  If other low grade oil is used, bearing bushing burning failure will occur in short time. Intensify diesel engine high grade oil, and its physical and chemical performance index can be identified by test, but relevant characteristics such as clean disparity, anticorrosion, anti-foam, oil membrane intensity cannot be checked by simple physical and chemical test. Moreover, it is required to judge if intensify characteristic loses, and this point is difficult to reach in some companies. So it cannot judge oil is in failure only by simple physical and chemical experiment, thus wrongly extend oil change period. It is better to change oil strictly according to oil change interval under different working conditions specified in user manual.

There’s another obvious sound, especially at idle, engine has “dang, dang” rhythmic sound, and when depressing clutch pedal, noise disappears. Generally speaking, it results from big axial gap of crankshaft when the crankshaft is making axial sudden movement.  At this moment, it is required to change crankshaft spacers.