Diesel engine shuts down automatically after start

Checking steps

Step 1: read flash code, identify the failure point; if fail to identify, turn to next step.

Step 2: check if low pressure fuel pipe is leaking air, or blocked (including return pipe), and whether inlet and exhaust is normal. If problems still exist, turn to next step.

Step 3: check if wires, sensors and plugs are damaged. If problems still exist, turn to next step.

Step 4: contact authorized service station, and have them test with special equipment as following steps.

Check by diagnostic instrument if flywheel signal disk and fuel pump cam signal disk are in synchronization (data flow test synchronization signal 48). If the problems still exist, turn to next step.

Further check by diagnostic instrument if rail pressure is built normally. If rail pressure is abnormal, maybe it is aroused by injector, common rail pipe, high pressure fuel pump and ECU. Please refer to relevant information.

Case 1:

Failure description

1. Diesel engine cannot be started up after it shuts down. There is no flash code.

2. Check and find out that there is no diesel in the fuel tank, then add diesel again and expel the air in low pressure fuel pipe. But it still cannot start.

3. Check high pressure fuel pipe, loosen two high pressure fuel pipe joints at fuel pump end, and start it with the starter, finding no fuel discharging.

It is confirmed that there’s air in high pressure pump. Fold fuel return pipe by hand, and restart the starter for air discharge utill fuel comes out the connector end,

Connect it again and it starts properly.

Fault causes and remedies:

Because of lack of fuel in the fuel tank, bubbles sprayed by returned oil is sucked into oil pipe, causing low pressure fuel pipe and high pressure pump to contain oil and air.

Add enough diesels, and eliminate failure according to case description

Case 2:

Failure description:

1. Diesel engine has weak power and then shuts down after running for a while; but get normal after stopping for a while. However the same symptom will occur again after a while.

2. Read flash code 242 - too high water temperature.

3. Check and find that water pipe is normal, Measure by diagnostic instrument and find that water temperature reaches 107℃ quickly after it is started, but the actually measured temperature is 65℃. It is confirmed that water temperature sensor is damaged; causing the ECU to obtain wrong water temperature signal, and the engine shuts down automatically when entering into self-protection mode load.

Fault causes and remedies:

When the sensor is damaged, it sends error signal to ECU, and ECU makes wrong judgment, and diesel engine enters into self protection. Replace water temperature sensor

Case 3:

Failure description

1. Diesel engine starts normally; diesel engine rotating speed decreases automatically, and then is off. Flash code lamp is on

2. Read flash code 324 – vehicle speed signal problem. The user feedback is that during troubleshooting vehicle speed sensor was touched due to faulty odometer.

3. After replacing the vehicle speed sensor, it is normal after test, and failure is eliminated.

Fault causes and remedies:

Vehicle speed sensor is damaged

Replace sensor or shield vehicle speed signal

Case 4:

Failure description

1. Diesel engine stops automatically after being started for 1-2 seconds, and diesel engine is shaking at the same time, then rotating speed decreases immediately when it is up to 540 rpm without flash code displaying.

2. Check low pressure fuel pipe and high pressure fuel pipe and there is no leakage or flattening on fuel return pipe.

3. Dismantle fuel return pipe and common rail pipe return pipe (PRV), and start vehicle, and then find that when there’s burst sound on diesel engine, fuel sprays out from fuel return pipe of common rail pipe.

4. When inspecting by diagnostic instrument, it is found that rail pressure rises to 440bar, and then falls to 280bar after checking, and then it waves between 280 and 320bar. It is confirmed that common rail pipe release valve is damaged

Fault causes and remedies:

Common rail pipe release valve in failure

Replace common rail pipe

Case 5:

Failure description

1. Diesel engine stops suddenly during running, and cannot be started again, and there’s no reaction when ignition switch is turned on.

2. Check and find that power is normal, power supply line is connected, but diagnostic instrument cannot communicate.

3. There’s no voltage on 1.04 pin by multi-meter, and it is confirmed that ECU is damaged.

Fault causes and remedies:

Connectors are not connected well, sealing member is in failure with water, or voltage is too high, etc, thus burning ECU

Replace ECU

Case 6:

Failure description

1. Diesel engine stops suddenly during running, but it can still start, however, engine may still stop suddenly without flash code displaying.

2. Check and find that the key switch is normal (when key switch is short-circuited or open-circuited, either may lead to the symptoms above)

3. Check that ECU power harness is normal (when power supply is open-circuited, it may lead to the symptoms above)

4. Check the auxiliary shutdown switch (shutdown switch under vehicle), and find that insulation rubber is worn out and earthed, thus ECU receives engine stop signal.

Fault causes and remedies:

Harness is not fixed well, and signal is in open-circuited when torn out, thus it sends an error signal to ECU

Connect the wire again and failure is eliminated