Case 1:

Diesel engine series: WP10.290

Failure time: 2008.02.18

Failure description

The starting of vehicle is controlled, put key switch to the position of T50, the starter does not work, and the vehicle cannot be started.

Failure reason analysis

It is doubted about the connection problem, ECU has no voltage input, or start relay malfunctions.

Measure 4 voltage input wires of connectors by a multi-meter, and find that there is no voltage; when checking vehicle circuit, it is found that a 30A fuse controlling ECU voltage is burned out; after replacement of new fuse, the vehicle starts normally.

Case 2:

Diesel engine series: WP10.336

Failure time: 2008.02.26

Failure description

Engine cannot be started, self-checking lamp does not work

Failure reason analysis

It is doubted that ECU has been burnt out

Check and find that 1.02, 1.03, 1.08, 1.09 and 1.40 voltage are 24V, it means there’s voltage input, but not sure about if there is communication with ECU or possibility of poor connecting. After further checking big plug harness, it is found that 1.40 pin is not completely plugged in, and the contact is not good. After it is well connected, the failure lamp is on. Meanwhile, K line voltage is 23 V, but INCA cannot be connected all the time. Check it again and find that the 8 wires of vehicle power are all well connected, so it can be concluded that ECU is burned out, and vehicle can be started normally after replacement of ECU and data renewal.

Case 3:

Diesel engine series: WP10.290

Failure time: 2008.03.16

Failure description

Malfunction lamp is always on, K line cannot be connected and the engine cannot be started

Failure reason analysis

Internal chip of ECU is burned out, and replacement of ECU is needed

Malfunction lamp is always on since delivery. The maximum rotation of engine is 1500 r. in such case, the driver drives the vehicle to Quxin service station, after checking, it is found that K line cannot be connected, malfunction lamp is always on, and diagnostic switch does not work. It is identified as ECU failure. Failure is eliminated after replacement of ECU

It is doubted that ECU has already been broken by welding in refitting plant.

Annex: Electric control system has many additional high sensitive parts, such as sensor, ECU internal chip, etc. These parts are liable to damage when exposed to high temperature, so it is recommended that vehicle manufacturer or refitting plant should pay more attention to high temperature protection.

Case 4:

Diesel engine series: WP12.336N

Failure time: 2008.03.16

Failure description

Vehicle does not display any failure code, and engine cannot be started even with starter starting several times, but when removing air in the low pressure and high pressure fuel pipes, it can be started smoothly. However, after switching off for about 5 minutes, it cannot be started again, or started in difficulty.

Failure reason analysis

It is doubted that low pressure fuel pipe is not sealed properly.

It has starting difficulty in factory, so inlet fuel pipe from fuel tank to coarse filter is replaced. The coarse filter on the passenger vehicle is usually designed for the convenience of fuel pump, which is moved behind the chassis. Besides, the connector of inlet end and outlet end of coarse filter are both designed by manufacturer, which is also critical part of checking. After dismantling coarse filter, it is found that there’s damage of screw on fuel inlet end of coarse filter, and inlet end has tiny crack. Because the sealing of low pressure fuel pipe is not proper, air enters low pressure fuel pipe, and establishment of rail pressure is difficult, representing difficulty to start or cannot be started.

Case 5:

Failure description

1. Starter is running normally, but diesel engine cannot be started.

2. Check low pressure fuel pipe, and find that fuel tank is waxed, so the fuel loses liquidity, and fuel pipe and filter core are blocked.

3. After inquiring the user, it turned out that -10＃ diesel was added when delivery of vehicle in , and the vehicle could not be started the next day when it came to Liaoning  and parked. Because the temperature in Liaoning was lower, -35＃ diesel should be added in .

Failure analysis and remedies:

Because vehicle users do not choose diesel with different grade number according to different environment temperature, diesel is waxed and fuel inlet pipe is blocked.

Vehicle running in areas with large temperature difference shall change diesel for the temperature according to actual situation

Get fuel pipe through without block, and replace diesel with -35＃diesel

Case 6:

Failure description

1. Starting system and low pressure fuel pipes are in good conditions; no flash codes are found.  The above methods still cannot help to eliminate it.

2. Use the diagnostic instrument to measure and find that rail pressure cannot reach the starting pressure of 160bar. Check and find that there’s no leakage on common rail pipe relief valve, and that the injector is in good conditions.

3. Disconnect the outlet fuel pipe of high pressure fuel pump, and run diesel engine with the help of the starter, then find that both outlets of high pressure fuel pump run fuel, but fuel height of one is4.5cm, and the other is less than 2cm. By visual comparison, it is found that the fuel supply of fuel pump is insufficient (diesel engine rotation speed is 200-250 r/min, the fuel height of high pressure fuel pump outlet is about 4-5cm, which is normal).

Failure analysis and remedies:

Poor quality diesel with high content of water causes damage of plunger piston of high pressure fuel pump

Replace the high pressure fuel pump, and install one more water filter.