5. Engine at unstable idle speed

◆ Causes and remedies:

①Injector is not working normally: Check injectors of all cylinder and harness.

②Vehicle with speed sensor has speed signal input when parking: check speedometer and speed sensor’s signal and wiring;

③Fuel quality is low with water or wax: Clean fuel system, and replace fuel filter.

④Low pressure fuel pipe has air leakage: check fuel pipes and connectors for air tightness; when necessary, expel air;

⑤Injection nozzle spraying is unstable: check and repair the nozzle

Note: When low water temperature causes idle speed increase, it is a normal function of ECU.

②In case of other loading (when air-conditioner is turned on), idle speed will increase by 100rpm naturally.

Remarks:

◆ Weichai Power GB 3 series diesel engine adopts Bosch generation II high pressure common rail system. High pressure common rail system has high requirement on fuel pipes (including low and high pressure fuel pipes). Many related problems are caused by fuel pipes.

◆Vehicle harness problem is also main source of failure

In case of fault, the two factors shall be considered at first

◆Common failure:

Vehicle speed sensor is damaged -〉cruise malfunctions, idle speed may be unstable;

The two sensors of brake pedal are not in synchronization-〉cruise malfunctions, and system lamp is always on

Clutch switch in failure-〉cruise malfunctions