Engine “cylinder bumping” accident

Engine “cylinder bumping” accident happens due to many factors. When piston reaches to TDC, in normal conditions, there’s only 1mm clearance to lower surface of cylinder cover, and any incorrect installation will cause piston “cylinder bumping” accident.

Firstly, if the assembler does not pay attention to timing mark during installation of camshaft gear or timing lines are marked in wrong place, it will cause wrong position of phase of valve open and close, and collision to opening valve when piston move to TDC, and it may even break piston and cylinder sleeve and bend valve, or in more serious case, damage cylinder body and other accident.

Secondly, incorrect installation during repair, such as big gap difference linkage, insufficient linkage bolt torque, reusing linkage bolt may cause linkage cover to disconnect, the linkage bolt to deform or break, consequently causing the piston “cylinder bumping”.  “Cylinder bumping” occurred due to too big linkage cover gap and loose cover etc results from inertial force when the engine is running, so the sound of “cylinder bumping” can be rather obvious only during running. It can also make terrible accident of damage cylinder body in more serious case.

When foreign objects enter cylinder, or parts of engine drop into cylinder (e.g., valve seat drops into cylinder, valve drops into cylinder), these are also causes of “cylinder bumping”

The sound of “cylinder bumping” of engine is clear and obvious. When there’s obvious sound of “Dar dar” on the cylinder cover, it is required to immediately stop running, and dismantle cylinder cover and check internal conditions. If there’s obvious mark of collision by other body on the top of piston, check that any foreign matter exists in the cylinder.  If there’s obvious mark showing that the valve knocks the top on the top of piston, it is required to check if valve gap and camshaft gear timing mark are correct. If it is found that large area on the top of piston is in collision with cylinder cover, the linkage bushing shall be checked.