Engine discharges black smoke:

Engine may discharge smoke when injection is too early (injection advance angle is big). If the black smoke limiter on injection pumps malfunctions, the engine may discharge black smoke as well when engine accelerates. In case of very dense smoke, it is required to consider other causes; sometimes unnoticeable and small failure will lead to serious fault symptom, such as engine shuts down and working cylinder does not react, then leading to dense black smoke.

Troubleshooting steps

Step 1: check if the air cleaner, intake/exhaust pipes and valve gap are normal; if normal, turn to next step

Step 2: check if the supercharger works normally. If normal, turn to next step

Step 3: check if diesel quality is normal. If normal, turn to next step

Step 4: Contact the technicians in the special maintenance station, and have them conduct the following steps by special equipment.

Check if the flywheel signal disk and fuel pump cam signal disk are in synchronization (data flow test synchronization signal 48) by diagnostic instrument. If still black smoke appears, turn to next step

Further check by diagnostic instrument if rail pressure is built normally. If rail pressure is abnormal, this fault might be caused by injector, common rail pipe, high pressure fuel pump and ECU, please refer to relevant information.

Case 1:

Failure description:

1. Diesel engine discharges black smoke suddenly during running process; the driver stop the vehicle immediately. It is hard to start later, and when starting, it discharges black smoke still.

2. everything seems normal after checking according to step 1, 2 and 3.

3. When it is started, rail pressure can only reach to 160bar with checking by diagnostic instrument

4. Check low pressure fuel pipe, common rail pipe, high pressure oil pump, and it is found that they are all normal; after checking the injector, it is found that the 2nd cylinder injector fuel return is abnormal, and it is estimated that the injector malfuctions.

Fault causes and remedies:

Injector needle valve torn out seriously, fuel spray is not good.

Replace the injector

Case 2:

Failure description:

1. Diesel engine discharges black smoke; after checking air cleaner elements, air pipes and the supercharger, it is found that they are in good condition.

2. Check low pressure fuel pipe and oil products, and it is found that diesel contains water. Dismantle injector, and find that there’s water rust in oil, and it is estimated that water in diesel has led to seizure and rust of 6 injector needle valves.

Fault causes and remedies:

Oil quality is unqualified, and oil contains water; and there’s no water filter, which caused rust and seizure of injector needle valve, and fuel spray is not good.

Replace it by acceptable diesel; replace the injector, mount one water filter

Case 3:

Failure description:

1. Diesel engine has black smoke when heavy loading, and there’s no black smoke when no loading, and no flash code is found. Stop the vehicle. After checking air cleaner elements, air pipes and the supercharger, it is found that they are in good condition.

2. Check and find that fuel pipe and oil quality have no problem.

3. Monitoring intake pressure, rotation speed, rail pressure, injection quantity by diagnostic instrument; it is found that when heavy loading is accelerating, air intake pressure can reach to 1.5kg (absolute pressure), and then decrease to0.6 kg quickly.

Rotation speed is decreasing with black smoke, and it is sure that there are some problems on air duct. After checking air pipes one by one, it is found that a rubber hose between air and air cleaner is sucked in flat when speed increase on high gears, leading to insufficient air intake of diesel engine.

Rubber hose in intake pipe is too soft, and it is sucked in flat when speed increase on top gear, leading to insufficient air intake of diesel engine.

Fault causes and remedies:

Replace the air intake hose