Generally, if white smoke still appears even when the engine starts, two reasons will cause it.

1) Injection is too late (injection advance angle is small) so that combustion is not sufficient;

2) Slight water leakage occurs in cylinder. Cold engine discharges white smoke; especially when it is initially started in winter, engine discharges white smoke, and it is normal that white smoke disappears as machine temperature rises.

Case of white smoke:

Diesel engine series: WP12.375N

Failure description

There’s white smoke phenomenon when it is started, and dense black smoke is discharged when depressing the accelerator pedal.

Failure reason analysis

Fuel pipe problem or air intake problem

There’s white smoke when it is started, and when depressing pedal, engine starts to discharge black smoke, then after reading flash code, it is found that there is no failure code, which indicates that engine data is correct. Next, analyze mechanical failure, and find that one section of intake pipe on intercooler is flat, see the picture:

It is obvious that the reason for the failure is that there’s block in the intake system, and it is serious. When engine is started, air intake is aspired by lower of air intake stroke piston itself, thus lead to low quality of combustible mixed air in cylinder, then insufficient combustion discharge white smoke, and then black smoke after accelerating.

In the beginning, it is doubted that the section between supercharger and intercooler is blocked; but after it is dismantled, there’s no stuff blocked inside, and there’s less possibility that inside of intercooler is blocked, then we checked air cleaner, and find that the secondary filter element is in wrong direction during installation, and installed one end of fixed plate in inner side, so plate has blocked the intake vent of air cleaner, thus engine intake is not enough seriously ,and combustion continues and without fuel, then white smoke appears.

See pictures attached.

The figure shows correct installation status after adjustment; when failure is not eliminated, filter element fixed plate is in the inner side to clog air inlet vent completely.