1. Read flash code according to flash code lamp to confirm failure point. If cannot confirm, turn to next step.

2. Check if the gear is at neutral position, if neutral gear switch is normal and the auxiliary shutdown switch (shutdown switch under vehicle) is normal. If engine still can not be started, turn to next step.

3. Check if starting harness and battery of the complete vehicle are normal. If engine still can not be started, turn to next step.

4. Check if the starter works normally. If the problem is still not solved, it is required to check if the engine is rotated smoothly and valve timing is correct, etc. If it still cannot start, turn to next step.

5. Check if the low pressure fuel pipe has air or air leakage or is clogged. If it still cannot start, turn to next step.

6. Contact workers in the authorized service station and use special equipment

Follow the steps to test: Check if the flywheel signal disk is synchronized (data flow testing synchronization signal is 48) with fuel pump cam signal disk by diagnostic instrument. If it still cannot start, turn to next step;

7. Further check by the diagnostic instrument if the rail pressure is building up normally; in case of abnormal rail pressure, maybe it is caused by injector, common rail pipe, high pressure fuel pump and ECU. Please refer to relevant information.