Engine is always running at 1000r.

Troubleshooting steps

Step 1: read flash code and determine failure point

Step 2: eliminate failure according to flash code

Step 3: check if the electronic accelerator pedal plug is loose, in wrong connection or short circuit

Step 4: check if the electronic accelerator pedal is in failure

Accelerator failure: ECU judges loading through signal from pedal potentiometer. For consideration of safety, when the pedal malfunctions, ECU controls diesel engine to return to an idle speed of 1000rpm automatically.

◆ Electronic accelerator pedal is loose or wrong connection

Plug connector again or check if accelerator wire is right, and then connect wire again.

◆ Electronic accelerator connector has water in

Blow connector with tools and then start

Notice: When replacing accelerator, the same model of accelerator is needed.

Failure case:

1. When diesel engine is started, rotation speed is always at 1000r, accelerator pedal is not working, and flash code lamp is on

2. Flash code is 221- accelerator pedal signal abnormal

3. Check if electronic accelerator pedal plug in is normal

4. Check accelerator pedal voltage by diagnostic instrument and twice of voltage is not right, and check and find that the wiring connection is normal.

Judgment of accelerator pedal sensor failure

Fault causes and remedies:

Accelerator pedal sensor failure, causing signal error

Replace the accelerator pedal