Engine “water back” failure

In actual work, we hear some feedback of “water back” on engine failure, and sometimes, “water back” symptom occurs even on new car.

Fig. 1-1 shows engine cooling system.

| 1.Heating radiator switch | 2. Warm radiator | 7.Roof cover of expansion tank |
| --- | --- | --- |
| 8.Expansion tank | 9.Thermostat port | 10. Water pump |

Fig. 1-1 Engine Cooling System

When filling engine with coolant (water or anti-freeze fluid), screw up the filler port of expansion tank, and add coolant to the level shown on glass window or fill directly to filling mouth, and then start engine. When engine accelerates, coolant will spill out from filler mouth, and many operators think this is “water back” failure on engine. In fact, it is normal phenomenon. Because coolant will expand during running of engine, currently many vehicles have been equipped with expansion tank to guarantee cooling effect and not increase volume of radiator tank, and eliminate air damping phenomenon by air during water circulation, and store water. According to normal requirement, when coolant is add to specific height, the filler lid shall be sealed, so that when engine is working normally, especially in high temperature or emergency speed-up, coolant level will be higher than filler level, leading to expansion of coolant. So when the filler lid is opened and the accelerator pedal is depressed with much force, coolant will spill out, which is normal phenomenon， and it is not “water back” failure.

The real “water back” failure occurs during running process of engine, coolant spills out of overflow pipe over expansion tank continually, when it stops spilling out and engine shuts down, So it is obvious that it is lack of water. “Water back” failure sometimes happens on hot car, and it is caused by overheat of engine or damage of cylinder gasket. Sometimes when engine temperature is not high, “water back” even occurs, This is due to partial block of coolant water tank, thus water tank water discharge ability becomes poor, and coolant from upper water chamber cannot flow into lower water chamber in time. At this time, it indicates that the radiator is required to be washed completely

In addition, heater water pipe needs to be checked, and see if it is connected wrongly.