# ECU Pin definition drawing and routine measurement

| Pin Number | Abbreviation | Pin specification | Referance pin | Voltage （ECU open circuit voltage） | Signal input Effective voltage | Current | Resistance |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 101 | dnc | Unused(the same below) |  |  |  |  |  |
| 102 | V_V_BAT+3 | ECU POWER SUPPLY +_3 | 1.05 | Battery voltage |  |  |  |
| 103 | V_V_BAT+4 | ECU POWER SUPPLY +_4 | 1.05 | Battery voltage |  |  |  |
| 104 | O_V_RH01 | Power postive output 1(high end drive) V2 | 1.05 | Battery voltage-0.26V |  |  |  |
| 105 | G_G_BAT-3 | ECU POWER SUPPLY -_3 | Body | 0V |  |  |  |
| 106 | G_G_BAT-4 | ECU POWER SUPPLY -_4 | Body | 0V |  |  |  |
| 107 | dnc |  |  |  |  |  |  |
| 108 | V_V_BAT+1 | ECU POWER SUPPLY +_1 | 1.05 | Battery voltage |  |  |  |
| 109 | V_V_BAT+2 | ECU POWER SUPPLY +_2 | 1.05 | Battery voltage |  |  |  |
| 110 | G_G_BAT-1 | ECU POWER SUPPLY -_1 | Body | 0V |  |  |  |
| 111 | G_G_BAT-2 | ECU POWER SUPPLY -_2 | Body | 0V |  |  |  |
| 112 | dnc |  |  |  |  |  |  |
| 113 | O_S_FAN2 | Fan relay 2 | 1.05 | Battery voltage-0.26V |  |  |  |
| 114 | dnc |  |  |  |  |  |  |
| 115 | dnc |  |  |  |  |  |  |
| 116 | dnc |  |  |  |  |  |  |
| 117 | dnc |  |  |  |  |  |  |
| 118 | dnc |  |  |  |  |  |  |
| 119 | dnc |  |  |  |  |  |  |
| 120 | dnc |  |  |  |  |  |  |
| 121 | dnc |  |  |  |  |  |  |
| 122 | O_S_DIA | Diagnosis LED | 1.05 |  |  | <125mA | <380mOhm |
| 123 | dnc |  |  |  |  |  |  |
| 124 | G_R_IIBTN1 | Idling lift switch 1，Inner grounding | 1.05 | 0V |  |  |  |
| 125 | I_S_IIBTN1 | Idling lift switch 1，Signal | 1.24 | 4.9~5.1V |  |  | 1.34~1.38KOhm |
| 126 | I_S_IIBTN2 | Idling lift switch 2，Signal | 1.05 | 4.9~5.1V |  |  | 1.34~1.38KOhm |
| 127 | dnc |  |  |  |  |  |  |
| 128 | G_R_IIBTN2 | Idling lift switch 2，grounding | 1.05 | 0V |  |  |  |
| 129 | G_R_DIG | Digit grounding | 1.05 | 0V |  |  |  |
| 130 | G_G_DIA | Inner grounding | 1.05 | 0V |  |  |  |
| 131 | I_S_CRCPOS | Cruise control button “Set/ acceleration” | 1.05 | 0~0.0102V | High-level：>3.98V Low-level：<1.97V |  | 4.06~4.19kOhm |
| 132 | I_S_ENGSA | Engine start switchSignal |  |  | High-level：>3.75V Low-level：<2.18V |  | 4.06~4.19kOhm |
| 133 | O_F_ENGN | Engine speed output signal | 1.05 |  |  | <50mA |  |
| 134 | B_D_CANL | CAN_L | 105/135 | 2~3V | High-level：>900mV Low-level：<500mV |  | 117~121Ohm |
| 135 | B_D_CANH | CAN_H | 1.05/1.34 | 2~3V | High-level：>900mV Low-level：<500mV |  | 117~121Ohm |
| 136 | dnc |  |  |  |  |  |  |
| 137 | O_S_STRTH | Start relay high-end | 1.05 | Connect to relay ： Not start: 3.5V Start :24V |  |  |  |
| 138 | O_S_CSLP | Cold-start lamp | 1.05 | 2.91~3.91V |  |  |  |
| 139 | O_S_WLP | Alarm lamp | 1.05 | 2.91~3.91V |  |  |  |
| 140 | I_S_T15 | T15 |  |  | ECU excitation voltage：3.35～24V Voltage of ECU from ON to OFF :0～2.81V |  |  |
| 141 | I_S_BRKMN | Main brake switch | 1.05 | 0V | High-level：>3.75V Low-level：<2.18V |  |  |
| 142 | I_S_ACSW | Air conditioning request switch | 1.29 | 0V | High-level：>3.75V Low-level：<2.18V |  |  |
| 143 | I_S_WFLS | Oil-water separation water level sensor | 1.05 | 4.9~5.1V |  |  |  |
| 144 | I_S_BRKPS | Parking brake | 1.05 | 4.755～5.25V |  |  |  |
| 145 | dnc |  |  |  |  |  |  |
| 146 | I_S_CRCRES | Cruise control button”reset” | 1.05 | 0～0.0102V | High-level：>3.98V Low-level：<1.97V |  |  |
| 147 | I_S_STP | Stop switch of engine | 1.05 |  |  |  |  |
| 148 | I_S_LIS | low idle switch | 1.05/1.29 | 5.53～5.9V | High-level：>3.75V Low-level：<2.18V |  |  |
| 149 | I_S_BRKRED | Redundant control switch | 1.05 | 0V | High-level：>3.75V Low-level：<2.18V |  |  |
| 150 | dnc |  |  |  |  |  |  |
| 151 | O_S_STRTL | Start relay low-end | 1.05 | Connect to relay： Not start:3.5V Start:0V |  |  |  |
| 152 | dnc |  |  |  |  |  |  |
| 153 | dnc |  |  |  |  |  |  |
| 154 | dnc |  |  |  |  |  |  |
| 155 | O_S_IMPH | Air intake pre-heating relay | 1.05 | Vbat－0.45V |  |  |  |
| 156 | O_S_WFLP | Alarm lamp for oil consisting of water | 1.05 | 2.91～3.91V |  |  |  |
| 157 | dnc |  |  |  |  |  |  |
| 158 | G_G_FAN2 | Fan relay2 grounding |  |  |  |  |  |
| 159 | G_G_IMPH | Air intake pre-heating relay grounding | Body | 0V |  |  |  |
| 160 | dnc |  |  |  |  |  |  |
| 161 | I_S_T50 | T50 Start switch | 1.05/1.59 | 0V | Input High-level：>3.61V Input Low-level：<2.33V |  |  |
| 162 | I_A_TL | Torque limited Signal （Multi-Mode switch ） | 1.65 | 4.9~5.1V |  |  |  |
| 163 | dnc |  |  |  |  |  |  |
| 164 | I_S_CRCNEG | Cruise control button “Set/ deceleration” | 1.05 | 0～0.0102V |  |  |  |
| 165 | G_R_TL | Torque limited inner grounding | Body | 0V |  |  |  |
| 166 | I_S_CONV | Clutch switch signal | 1.05 | 0V | High-level：>3.75V Low-level：<2.18V |  |  |
| 167 | G_R_FSS | Fan speed sensor grounding | Body | 0V |  |  |  |
| 168 | V_V_5VFSS | Fan speed sensor Power supply | 1.67 | 4.9~5.1V |  |  |  |
| 169 | I_F_FSS | Fan speed sensor Signal | 1.67 | 4.84～5.16V | High-level：>3.69V Low-level：<2.25V |  |  |
| 170 | G_R_VSS | Speed sensor grounding | Body | 0V |  |  |  |
| 171 | I_F_VSS | Speed sensor Signal | 1.70 | 4.05～4.34V |  |  |  |
| 172 | I_S_DIAREQ | Diagnosis request switch | 1.05 | 0～0.0102V | High-level：>3.69V Low-level：<2.25V |  |  |
| 173 | dnc |  |  |  |  |  |  |
| 174 | I_S_CRCOFF | Cruise control button “OFF” | 1.05 | 0～0.0102V | High-level：>3.98V Low-level：<1.97V |  |  |
| 175 | dnc |  |  |  |  |  |  |
| 176 | G_R_APP2 | Position sensor of accelerator pedal2 grounding | Body | 0V |  |  |  |
| 177 | V_V_5VAPP1 | Position sensor of accelerator pedal1 Power supply | 1.78 | 4.9~5.1V |  |  |  |
| 178 | G_R_APP1 | Position sensor of accelerator pedal1 grounding | Body | 0V |  |  |  |
| 179 | I_A_APP1 | Position sensor of accelerator pedal1 Signal | 1.78 | 0V | Not stamp0.75V When pedal stamp in the end3.84V |  |  |
| 180 | I_A_APP2 | Position sensor of accelerator pedal2 Signal | 1.76 | 0V | Not stamp0.375V When pedal stamp in the end1.92V |  |  |
| 181 | I_A_FFDPS | Fuel filter differential pressure transducer | 1.83 | 5.75~5.85V |  |  |  |
| 182 | dnc |  |  |  |  |  |  |
| 183 | G_R_FFDPS | Fuel filter differential pressure transducer grounding | Body | 0V |  |  |  |
| 184 | V_V_5VAPP2 | Accelerator pedal position sensor 2 power supply | 1.83 | 4.9~5.1V |  |  |  |
| 185 | I_S_GNSW | Neutral switch | 1.05/1.29 | 0~0.0105V |  |  |  |
| 186 | dnc |  |  |  |  |  |  |
| 187 | dnc |  |  |  |  |  |  |
| 188 | dnc |  |  |  |  |  |  |
| 189 | B_D_ISOK | K WIRE | 1.05 | Battery voltage |  |  |  |
| 201 | O_T_VTG | Fan solenoid valve | 1.05 | 2.91~3.91V |  |  |  |
| 202 | dnc |  |  |  |  |  |  |
| 203 | O_V_RH02 | Power supply postive output 2 (high-end drive) | 1.05 | Battery voltage |  |  |  |
| 204 | O_S_FLHT | Fuel heating solenoid valve | 1.05 | Vbat－0.45V |  |  |  |
| 205 | G_G_FLHT | Fuel heating solenoid valve inner grounding | Body | 0V |  |  |  |
| 206 | O_T_EBRFL | Air exhaust brake solenoid valve | 1.05 | 2.91~3.91V |  |  |  |
| 207 | dnc |  |  |  |  |  |  |
| 208 | dnc |  |  |  |  |  |  |
| 209 | I_F_CAS | Camshaft speed sensor signal | 2.10 | 231~260mV |  |  |  |
| 210 | G_R_CAS | Camshaft speed sensor grounding | 1.05 | 0V |  |  |  |
| 211 | O_S_ACRLY | A/C compresser relay | 1.05 | 2.91~3.91V |  |  |  |
| 212 | G_R_RAILPS | Rail pressure sensor grounding | 1.05 | 0V |  |  |  |
| 213 | V_V_5VRAILPS | Rail pressure sensor power supply | 2.12 | 4.9~5.1V |  |  |  |
| 214 | I_A_RAILPS | Rail pressure sensor signal | 2.12 | 4.9~5.1V |  |  |  |
| 215 | I_A_CTS | Cooling water temperature sensor | 2.26 | 4.9~5.1V |  |  |  |
| 216 | V_V_5VRMTAPP1 | Remote accelerator sensor1 power supply | 1.05 | 4.9~5.1V |  |  |  |
| 217 | G_R_5VRMTAPP1 | Remote accelerator sensor1 grounding | Body | 0V |  |  |  |
| 218 | G_R_5VRMTAPP2 | Remote accelerator sensor2 grounding | Body | 0V |  |  |  |
| 219 | G_R_CRS | Crankshaft speed sensor grounding | 1.05 | 0V |  |  |  |
| 220 | dnc |  |  |  |  |  |  |
| 221 | I_A_RMTAPP1 | Remote accelerator sensor1 signal | 2.17 | 0V | Not stamp0.75V When pedal stamp in the end3.84V |  |  |
| 222 | I_A_RMTAPP2 | Remote accelerator senser2 signal | 2.18 | 0V | Not stamp0.375V When pedal stamp in the end1.92V |  |  |
| 223 | I_F_CRS | Speed sensor signal of crankshaft | 2.19 | 231~260mV |  |  |  |
| 224 | G_R_OPS | Oil pressure sensor grounding | Body | 0V |  |  |  |
| 225 | G_R_BPS | Air intake pressure, temperature sensor grounding | Body | 0V |  |  |  |
| 226 | G_R_CTS | Cooling water temperature sensor grounding | Body | 0V |  |  |  |
| 227 | I_A_OPS | Oil pressure sensor signal | 2.24 | 5.243~5.406 |  |  |  |
| 228 | I_A_OTS | Oil pressure sensor signal | 2.24 | 4.9~5.1V |  |  |  |
| 229 | I_S_EBR | Air intake exhaust switch | 2.17 | 0V | high-level：>3.57V Low-level：<2.35V |  |  |
| 230 | dnc |  |  |  |  |  |  |
| 231 | V_V_5VRMTAPP2 | Remote accelerator sensor2 power supply | 217 | 4.9~5.1V |  |  |  |
| 232 | V_V_5VOPS | Oil pressure sensor power supply | 217 | 4.9~5.1V |  |  |  |
| 233 | V_V_5VBPS | Air supercharge pressure sensor power supply | 217 | 4.9~5.1V |  |  |  |
| 234 | I_A_BPS | Air supercharge pressure sensor signal | 2.25 | 5.243~5.406 |  |  |  |
| 235 | I_A_FTS | Fuel temperature signal | 2.26 | 4.9~5.1V |  |  |  |
| 236 | I_A_BTS | Air intake temperature sensor signal | 2.25 | 4.9~5.1V |  |  |  |
| 301 | O_P_SVH21 | Electric capacity high end 2 of fuel injector 1 | 1.02 |  |  |  |  |
| 302 | O_P_SVH22 | Electric capacity high end 2 of fuel injector 2 |  |  |  |  |  |
| 303 | O_P_SVH23 | Electric capacity high end 2 of fuel injector 3 |  |  |  |  |  |
| 304 | O_P_SVH11 | Electric capacity high end 1 of fuel injector 1 |  |  |  |  |  |
| 305 | O_P_SVH12 | Electric capacity high end 1 of fuel injector 1 |  |  |  |  |  |
| 306 | O_P_SV13 | Electric capacity low end 1 of fuel injector 3 | 1.05 |  |  |  |  |
| 307 | dnc |  |  |  |  |  |  |
| 308 | dnc |  |  |  |  |  |  |
| 309 | O_V_MEU | Flow metering unit power supply（BAT+） | 1.05 | Vbat－0.26V |  |  |  |
| 310 | O_T_MEU | Flow metering unit power supply PWM | 1.05 | 2.91~3.91V |  |  |  |
| 311 | O_P_SVH13 | Electric capacity high end 1 of fuel injector 3 |  |  |  |  |  |
| 312 | O_P_SV12 | Electric capacity low end 1 of fuel injector 2 |  |  |  |  |  |
| 313 | O_P_SV11 | Electric capacity low end 1 of fuel injector 1 |  |  |  |  |  |
| 314 | O_P_SV23 | Electric capacity low end 2 of fuel injector 3 |  |  |  |  |  |
| 315 | O_P_SV22 | Electric capacity low end 2 of fuel injector2 |  |  |  |  |  |
| 316 | O_P_SV21 | Electric capacity low end 2 of fuel injector1 |  |  |  |  |  |