Fuel consumption is too much

Air intake pipe is blocked (air cleaner is blocked)

Exhaust back pressure is too high

Fuel quality is too poor

Fuel pipe is blocked

There is oil leakage in fuel pipe

Air or fuel supply timing is not correct

There is oil leakage in cylinder pad.

Bearing bush space is too much, and engine needs an overhaul.

Piston expands out the cylinder

Pressure of supercharge system is insufficient

Supercharger malfunctions

Inter-cooler is damaged or has gas leakage.