Troubleshooting steps:

Step 1: check that the fault results from plateau and extreme-cold environment.

Step 2: read flash code and identify the failure point; if there’s no flash code, turn to next step

Step 3: check if starter is normal and battery quantity is sufficient; if it still cannot start successfully, turn to next step

Step 4: check if low pressure fuel pipe is normal; if it still cannot start successfully, turn to next step

Step 5: Further check if diesel engine can run smoothly, and if air supply timing is normal; if it still cannot start successfully, turn to next step

Step 6: check tif crankshaft rotating speed sensor and cam rotating speed sensor and plugs are damaged, firm or clean, if it still cannot start successfully, turn to next step

Step 7: contact authorized service station staff, and test as following steps by special equipment.

Check by diagnostic instrument if flywheel signal disk and fuel pump cam signal disk are in synchronization (data flow test synchronization signal 48). If it still cannot start successfully, turn to next step.

Further check by diagnostic instrument if rail pressure is built in normal; if rail pressure is abnormal, it is maybe because of injector, common rail pipe, high pressure

Start in difficulty failure case

Case 1:

Failure description

1. There’s no problem after failure is eliminated by elimination steps.

2. Check synchronization signal by diagnostic instrument, it can show 48, but responce time is too long (more than 3 seconds), check crankshaft rotating speed sensor, it is found that the surface of sensor and flywheel signal hole are dirty, then clean and install them again, the failure is eliminated.

Fault causes and remedies:

Foreign material causes that sensor cannot recognize signal hole location on flywheel accurately. When it is cleaned, it guarantees that sensor can be recognized correctly

Case 2:

Failure description

1. Diesel engine is difficult to start, and there’s big sound of “kaka” at the moment of engine starting. When the vehicle is started, rotating speed can only reach 1500r/min, and diesel enters into Limping Home state, and flash code lamp is on.

2. Read flash code 112- crankshaft rotation sensor is in failure, and after checking, it is found that crankshaft rotating speed sensor plug is loose. Failure is eliminated after reinstallation.

Fault causes and remedies:

Connector is not contacted well (it is possible that the sensor is forgotten to connect or the sensor malfunctions), so connect the sensor again (or replace with new sensor)